A Breviary for Electrical Engineers

A small compilation of electrical engineering material

that seems not ever to get communicated,

but that is expected to be known.

Started 15 February 2022 by David Kazdan and the Case Amateur Radio Club W8EDU

A long time ago, I attended a talk by Isaac Asimov.  He mentioned that in the 1930s, he kept asking his high school teachers to show him how to use a slide rule.  They kept telling him not now, you’ll be taught that when you need it.  He showed up for college chemistry and was given a messy bit of arithmetic to do.  He did it laboriously by hand, and the professor asked why he wasn’t using a slide rule.  He said he thought he would be taught how when he needed it.  The professor said we’re not going to teach that to you, you’re expected to know that by now, they should have taught it to you in high school.

Here are some items maybe you should have been taught but need to catch up on now.

–DK

Sinusoids

Sine with amplitude 10, frequency 2.5 Hz, phase pi/4. Note where the sinusoid peaks.

Peak-to-peak amplitude is 2*A; A=10 on this graph.  Root-mean-square (RMS) (“heating equivalent”) amplitude is 7.07 amperes, not marked on the graph. RMS is the voltage given for line current, such as 120 VAC, 60 Hz in the United States.

Sinusoids are interesting signals for a variety of reasons:

- Their energy is contained at a single frequency

- They have a complete set of integrals and derivatives

- They can encode information in a variety of ways: Amplitude, phase

- They have a good sensory appeal in sound, speech, and music

- They look cool on an oscilloscope both alone and in Lissajous figures

- They can change in both time and in space.  This one is viewed at a point in space, there’s no spatial variable in its equation and no consideration of speed of travel.

If your signal is known to be a sinusoid, that signal is completely defined by three parameters:

- Amplitude A (might be volts or amperes in a circuit; meters of displacement for vibration, pressure change from ambient for acoustics, population in epidemiology, and so on).

- Frequency f (common, or circular frequency) or ω (angular frequency, ω=2πf).

- Phase ϕ, the offset from cosine’s zero phase (see below). Don’t ignore this one.

  - Call a wave of this type a “sinusoid” when phase isn’t known or doesn’t matter.

  - Call it a sine or a cosine if phase is known relative to some signal of known phase or some time that can be called “time t=0”.  That timing may have a degree of arbitrariness to it; cf. Buffett 2003.

- In full form, with those three parameters as constants: s(t)=Acos(2πft+ϕ)

  - Use cosine form rather than sine form; it’s an even function, it’s purely real when the wave is given in analytic form (more below) and it’s the engineering standard.

  - ϕ of -90°, (or of -π/2), gives a sine function.  Sine lags cosine (look closely at the graphs.  Cosine has its positive-going zero crossing before time t=0; sine’s is at t=0, which is later), and the two are said to be “in quadrature.”

  - ϕ of +90°, or of +π/2, gives a negative sine function. This leads cosine and is also in quadrature with it.

Note that any or all of those three parameters may be functions of time. Information is encoded on the basis of those three parameters being changed with time: On-off telegraphy; AM vs. FM radio, phase modulation, multifrequency shift keying and “quadrature amplitude modulation” for data, and so on.

- If A is modulated, the sinusoid is said to be “amplitude modulated.”  Seems like an obvious tautology but there are several subspecies of AM to be considered.

- If f is modulated, the sinusoid is said to be “frequency modulated.” Keep track of the “instantaneous frequency” in that case.  Seems like an even more obvious tautology but again–there are several subspecies and they are linked to…

- “Phase modulation.”  Connected to the above by an integral/derivative relation.  Instantaneous phase, then, is the position of the cosine in its circular path.  See the next section about frequency being the time-derivative of phase.

Zero-crossings encode all the information in a sinusoid if the amplitude is either

known,

unchanging,

or irrelevant.  That last is the stuff of the noise-immunity of frequency modulation.

The National Institute of Science and Technology (NIST) has material on time and frequency issues here.

An example offered by Jonathan Jacobs PhD:

“To calculate phase you need to know the duration of a cycle. For example, at 0.5 Hz, one cycle takes two seconds. Since phase [difference] is a relative measurement (i.e., the value of one thing with respect to another) you must select some equivalent point on the eye and target plots, such as when they cross zero. In the following figure, the eye is behind (lag) the target by 0.25 seconds, which is one-eighth of a complete cycle, or 45°.

“(NOTE: This has been a historically confusing question in student lab reports. Why? Probably because of our culture’s left-to-right interpretation of progression: at first glance at the figure below, you might be tempted to say that the eye is ahead of the target since its trace appears to the right, making it look like it is winning a footrace. To clear up the confusion, look at the points indicated by the arrows. Observe that the target crossed zero at 0.75 seconds while the eye didn’t reach that same position until 1.0 seconds.)”

More on Sinusoids: Phase

The term has two different meanings in these arts (.i.e., it is “polysemous”), and workers are expected to get the correct meaning from context.

- “Phase” is the word used for the argument of a trig function. In s=cos(π/2), the “phase” of the cosinusoid is π/2.  This term relates to “phase” meaning “face,” as in the part of the moon’s face visible from earth. Instantaneous phase is the position of the cosine vector in the circle.  See this Wikipedia article on phase.

- The word can also mean the difference in that argument between two sinusoids; also called “phase difference” or “phase lag or lead” or “relative phase.” cos(x) leads sin(x) in the sense that it starts 90 or π/2 radians before.  Similarly, sin(x) is said to lag cos(x).

  - This meaning is made more explicit by always using cos(x) as the “standard” phase, or “zero” phase signal at a given frequency.  Other sinusoids of different phase are then made up of adding a phase offset to the cosine’s argument. s(t)=cos(x - ϕ), for example, lags s(t)=cos(x) by ϕ (look at the overall argument of the cosine, see that the argument is zero when x has progressed to ϕ radians).

  - Cosine is convenient as the “standard” of zero phase because it is an even function, while sine is odd.  It is the electrical engineering convention for zero phase, but not all other disciplines’.

Look at a time-dependent cosinusoid with constant frequency:

s(t)=cos(ωt)

The phase in the sense of “argument to the trig function” is ωt.  The phase by that definition changes linearly with time–so the time-derivative of phase is ω:

d(ωt)dt  = ω

As phase increases past 2π, that quantity (2π) is subtracted off to keep phase between 0 and 2π (or the subtraction is done to keep phase between -π and +π, depending on the context).  Continuously increasing phase, then, winds up looking like a sawtooth on a graph.  Each successive removal of a 2π makes the graph look like it has discontinuities but it really doesn’t. Decide if you like to use finite-height graph paper or not.  One way gives a smooth graph, the other gives the apparent discontinuities.

Frequency is the time-derivative of phase! This will be more apparent in the next sections, when we talk about exponential representation of sinusoids, sinusoids in time as rotating vectors, and phasors.

(similarly, wavenumber is the spatial-derivative of phase for traveling waves)

More on Sinusoids: Specifying Amplitude

A mathematician’s cosine has unitless amplitude 1, with a range of -1 to 1.

An engineer’s cosine usually multiplies that by something with units, for example 1 volt or 1 ampere.  Now the range is -1 volt to +1 volt.  You might say that its amplitude is 1 volt, but that turns out to be ambiguous.  Always specify one of these, or another measure of amplitude:

VP-P is volts peak-to-peak.  Maybe it should be called “trough to peak,” but it’s not.  The signal in the previous paragraph is 2 VP-P and this is a common way of specifying audio signals.  The usual audio output for computers and other consumer devices is (maximum) 1 VP-P.

VRMS, or “root-mean-squared,” is the average squared value of the wave.  It’s useful in examining power in a voltage or current sinusoid.  Into a constant resistance load, instantaneous power is proportional to the square of the voltage or current; the average of that power over a cycle gives the heating that would be equivalent to a DC source’s.  This only makes sense if the thermal time constant of the heater is long compared with the signal period.  In Edison’s day, the method would have been applied to an electric toaster (thermal time constant of, say, ten seconds) against the line current (power cycle period of 1/120 seconds).  Be careful that average over a time period is actually what you want. For a sinusoid, peak voltage and RMS voltage have a ratio of sqrt(2).  For other waveforms, the calculation may have to be done numerically or by actually measuring heating.

VRMS is useful in radio work, as the RMS voltage delivered to the antenna load measures the power emitted from the transmitter into free space by the antenna.  The time constants are the frequency of the radio signal and the signal bandwidth, so the comparison mentioned above holds and the measure is useful.  Peak voltage at the antenna terminal, however, will determine circuit parameters such as whether the output capacitor will arc over.

More on Sinusoids: Exponential Notation

This looks harder than it is, and it is fiendishly useful.  You may want to learn it and get accustomed to using it.  It starts with Euler’s equation,

ejx=cos(x)+jsin(x)

(the proof, by the way, is lovely and well worth an evening to fuss out.  If you like such things, try examining the Taylor expansion of the exponential function with an imaginary argument.  Another interesting and less obvious proof examines the derivative of the ratio of the two sides of Euler’s equation)

As a sinusoidal signal evolving in time, Eulder’s equation looks like this (substitute ωt for x):

ejωt=cos(ωt)+jsin(ωt)

You don’t need this, you can always separate out the even and odd parts of a sinusoidal signal as

se(t)=cos(ωt)	(the even part)

and

so(t)=sin(ωt)	(the odd part)

therefore

s(t)=se(t)+so(t)(the entire signal)

But if you use the “tagging” of the odd (sine) part with j, you get to carry the two together as a vector quantity.  The vector behaves a little like a programming object–one variable name denotes both parts of the signal simultaneously.

Even better, adding a phase offset lets you use only the real part for computations:

s(t)=Re(ej(ωt+ϕ))=cos(ωt+ϕ).  If ϕ=π/2, that real part is a sine, lagging 90 degrees behind the cosine, and it gets expressed as the real part of the exponential.

(There’s a definitional subtlety here. The real part of the complex number is real (no surprise:

ℜ(a + bj)=a, a real number (and yes, that Fraktur typeface is what’s used in print))

But note that “the imaginary part” of that same complex number,

ℑ(a+bj)=b, also a real number.  Again, the imaginary part of a complex number is a real number;  ℑ(a+bj) is not equal to bj, it’s b.) (also Fraktur. That’s an “I”, not an “F” and don’t look at me, I didn’t make this stuff up)

In most engineering computations, only the real part is used and the ℜ operator is assumed.  Voltage or current might be sinusoids but they can only be real, not imaginary; if a circuit shifts a cosinusoid to a sinusoid, the phase offset term will make the sinusoid come out in the real wash.  Some texts mention that in passing.  Many don’t; the authors assume that this knowledge was gained from previous reading.  All right, then: You read it here first!  Asimov had a point.

Linear systems in general take in a sinusoid of some amplitude, frequency, and phase, and output a sinusoid of the same frequency.  A linear system may change the input’s amplitude and phase in a frequency-dependent way, but cannot generate new frequencies.  Nonlinear systems can and do generate new frequencies, but linear ones cannot and don’t.  Linear systems’ changes in signal amplitude and phase have a good graphical representation called “phase vectors” or  “phasors.”  These are the next interesting aspect of exponential representation of sinusoids–and again, the word is used in two different ways.

Phasors: Two different meanings, you need to check context

Note again the expression for a single-frequency signal, s(t)=Vej(ωt+ϕ).  This is called the “analytic representation” of cos(ωt+ϕ).  The taking of the real part is assumed, so the expression is “really” ℜ(s(t)=Vej(ωt+ϕ)).

This expression may be interpreted as a vector rotating in the complex plane, or Argand plane.  With frequency ω positive, the vector rotates counterclockwise at that rate in radians per second.  In full rotations, the “normal frequency” f is ω/2π.  The angle at which the vector begins its journey, at time zero seconds, is ϕ.  The vector magnitude is V.  All three of V, ω, and ϕ may be functions of time or may be constants.  Time itself marches along, and that’s why the vector rotates.

Not a bad image of a phasor, but note that frequency ω is given in rads/s but both the phasor and the sinusoid’s phases are shown in degrees.  Oh, well.  Power engineers.

The first definition of “phasor” is the rotating vector as just described.  Time progresses, argument=ωt progresses with it, and the vector rotates. The rotational speed may vary; the vector length (signal amplitude) may vary with time as the vector rotates. The vector has length V and an offset at time t=0 of ϕ.  This definition is fine but it’s not the more common of the two.

The other, more common definition requires some more discussion:

By the rules of exponential arithmetic, the exponential terms may be separated:

s(t)=Vejϕejωt

The second definition of “phasor” is the stuff that multiplies the rotating part, just

phasor=Vejϕ

This version of the phasor has no frequency information and does not rotate with time unless the overall signal is angle-modulated.  A linear, time-invariant (LTI) system can change the amplitude and phase of an input signal in a frequency-dependent way, but it can’t change the signal’s frequency.  The signal as it manifests in different parts of the system may be examined with this version of the phasor.  A cosine has a phase of zero and some amplitude in this representation, regardless of its frequency.  As ϕ increases, the sinusoid leads the cosine.  If the angle (phi) is negative, the signal is lagging the cosine.  There’s no way around it, this gets confusing.  Draw some pictures, ponder some capacitors and inductors, get it right.

In signal representation such as AM and FM radio, the phasor is the incoming signal as heterodyned by the local oscillator.  If the signal has no frequency or phase change and is modulated in amplitude only, the angle depends on the relative phases of the incoming signal and the local oscillator but it stays put.  The vector magnitude changes.  Phase modulation sees the phase angle changing in proportion to the modulating signal; frequency modulation sees the derivative of the phase changing in proportion to the modulating signal.

Even and Odd Functions

Apropos the above material, it’s easy to prove (by construction) that every real function is separable into an “even” part that is symmetric across the y-axis, and an “odd” part that is antisymmetric.  An interesting function to explore in that way is the exponential function.  Every scientific calculator has a button for computing y=ex, and a separate pair of buttons for computing the even and odd parts of y=ex for x real.  Those buttons are labeled “hyperbolic cosine” and “hyperbolic sine,” respectively.  They don’t see a lot of use in contemporary electrical engineering but the functions do show up in structural analysis.

Those same calculators do not have a button for computing y=ex for x imaginary.  There are, however, buttons for finding the even and odd parts (both real numbers) of the exponential function for that case (x imaginary).  They are labeled “cosine” and “sine.”  Calculators such as the TI-84 which have complex numbers as a native datatype will permit that exponentiation, and will report back a complex number with the real part and the complex part multiplied by the imaginary unit.  There are some machinations to be done, description here and here. I am reasonably certain you can use j instead of i, but I’m not sure.  I use H-P calculators.

Work out the equations for each of these and verify the statements–but honest, they’re true!

Frequency: Definitions narrow and generalized; transforms Fourier and Laplace

We all learned that a Fourier transform integral has a jω in it, and a Laplace transform has an s instead.  I have yet to talk with an undergraduate electrical engineering student who can explain in a small number of grammatically correct sentences what s represents.  A handful can do it for jω.  Frequency is the “basis function” system for both, but “frequency” is defined as a purely imaginary number in one, and as a complex number in the other.  This is not as bad as it sounds: The imaginary part of the frequency encodes a sinusoid; the real part (absent in Fourier transforms) encodes an exponentially decaying or expanding envelope.

Frequency, in usual usage, has meaning only for periodic functions.  With a period of T (seconds, say), the frequency is 1/T (hertz, if the period is seconds).  In many settings, radian frequency is easier to use; that will be ω=2πf, or equivalently ω=2π/T.  That’s what is used in Fourier transforms, where the periodic signal is assumed to have started at time negative infinity and continues to time infinity. ω is the number that multiplies j and is the (imaginary) frequency.

OK, that note being played on a flute is periodic: First-register A (A above piano middle-C) is mostly a sinusoid at 440 hertz.  The signal satisfies the periodicity criterion that there is some time T you can shift the signal and it will superimpose on itself.  T is the period, which is the reciprocal of the frequency.  The flutist did start blowing at some time in the past and we expect that the blowing will stop at some time in the future, but right now, the start and end times are approximately infinitely distant.  Flutists learn good breath control.

The flute’s frequency is ω=2πf.  If f is 440 cycles per second (hertz), ω is about 2764.6 radians per second. That’s the frequency; in analytic form, it is the imaginary part of jω (the real part is zero).  jω, then, is the imaginary frequency and it encodes a sinusoid (without a phase offset from a cosine).

The same note plucked on a guitar isn’t periodic.  It is loudest at the moment of the pluck and it dies off at some exponential rate.  There must be some relationship between the flute’s note and the guitar’s, as you can tune one of the instruments to the other.  There isn’t any part of the guitar’s signal that is superimposable on another part.  It has a jω but it has something else, too.  Its “frequency” includes a decay term.  The generalized frequency is the s=σ+jω in the Laplace transform, which is a complex number.  The imaginary part of s is ω, the same as the imaginary frequency for the Fourier transform. It marks the frequency of the sinusoidal part, if there is one (ω may be zero).  Again, that frequency has units of radians per second; radians are dimensionless, so ωt is dimensionless and is a permitted argument for a transcendental function.  The real part of s is σ, which marks the decay.  It has units of nepers per second, which indicates the rate of the decay (or growth).  The same dimension discussion applies for nepers as for radians.  The entire complex frequency s encodes the rate of zero crossings if ω is nonzero, and the rate of decay or growth if σ is nonzero.

In linear systems, signals can be a sinusoid with exponential decay or growth.  The guitar’s has decay.  The flute’s is right at the boundary of growth and decay–it has constant amplitude until the next dynamic marking or breath.  Signals in some systems (not acoustic ones) can have the exponential decay or growth without having the sinusoidal part.  They don’t have periodic zero crossings.  For all of these except the non-decaying/non-growing sinusoid:  What is the frequency if there is no periodicity? By the narrow definition that frequency requires a repetition of the signal over some identifiable period, there is no frequency in a nonperiodic signal.  There is something else going on in signals that grow or decay, and it’s that the periodicity is contained in the zero crossings if the imaginary part of the (extended definition) frequency is nonzero.  That’s the guitar string.

In unilateral Laplace transforms, where functions are assumed to be zero up to time t=0 and then start doing whatever they’re going to do, a generalized version of frequency is used.  Your signals and systems text did mention this early on, but it gets lost in the shuffle so let’s look in some more detail now.

The complex frequency s is what shows up in the Laplace transform.  As do all complex numbers, it has a real part which may be zero, and an imaginary part which similarly may be zero.  As in the frequency definition used for Fourier transforms and phasors, the imaginary part encodes frequency in the periodic, sinusoidal sense.

Complex frequency is an extension of the usual concept of frequency, the one that involves periodicity.  In complex frequency, the periodicity may only be in the zero crossings.  That’s the imaginary part; it that part is zero, there are no zero crossings in the resulting signal.  The real part encodes an envelope, either increasing or decreasing exponential or constant, if the real part is zero.

Highly recommended reading on the distinctions between Fourier and Laplace transforms and phasor analysis is Fawwaz Ulaby et al.’s Circuit Analysis and Design,

Section 13-7 “Phasor vs. Laplace vs. Fourier,” p. 710 ff, Michigan Publishing 2018, available for download here.

Items on the Discrete Fourier Transform (DFT)

The DFT is the computationally possible, windowed version of the discrete-time Fourier transform, the DTFT.  The DTFT extends -∞ to +∞, just as does the usual (continuous time) Fourier transform. DFTs examine a finite number of data samples, and overall behave as though they are one cycle of a repeated signal.

Please call the DFT just that.  The fast Fourier transform (FFT) is an algorithm for efficient computation of the discrete Fourier transform (DFT), it’s not an independent transform.  If you want the bank of discrete bandpass filters that gets performed on a finite-length vector of sampled data, you want the DFT.  For any but the very shortest data sets, you’ll likely compute it with the FFT, but you are using the FFT to compute the DFT, not “computing an FFT.”.

Note that the DFT computed by itself, without the FFT, is a straightforward linear algebra problem.  A column vector of n time-domain data (your sampled data, usually real numbers from a microphone or some such) get multiplied by an n x n square matrix of complex coefficients to give an n long vector of complex frequencies.  This is well into the category of “it’s not as bad as it sounds.”  Look at the rows of the multiplying matrix:

| 1 | 1 | 1 | … | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
| 1 | ej2π/n | ej2π(2)/n | … | ej2π(n-2)/n | ej2π(n-1)/n |
| 1 | ej2π(2)/n | ej2π(2)(2)/n | … | ej2π(2)(n-2)/n | ej2π(2)(n-1)/n |
| … | … | … | … | … | … |
| 1 | ej2π(n-2)/n | ej2π(n-2)(2)/n | … | ej2π(n-1)(n-1)/n | ej2π(n-2)(n-1)/n |
| 1 | ej2π(n-1)/n | ej2π(n-1)(2)/n | … | ej2π(n-1)(n-2)/n | ej2π(n-1)(n-1)/n |

As an example, in the case where n=4, the DFT matrix would be

| 1 | 1 | 1 | 1 |
| --- | --- | --- | --- |
| 1 | j | -1 | -j |
| 1 | -1 | 1 | -1 |
| 1 | -j | -1 | j |

Each row represents an oscillator running at one specific frequency.  The real part is the sampled cosine at that frequency; the imaginary part is the sampled sine.  The zeroth row is at frequency f=0, or 1 all the way across.  The imaginary part is zero for all of them.  The resulting dot product with the data vector is the data average, or the DC component.  That’s what goes into frequency bin zero (after being normalized by 1/n).

The first row (first one after the zeroth row) is the unit vector rotating around the unit circle exactly once.  The number of steps of the rotation is the same as the number of samples in the data vector.  If your system is sampling at 8000 times per second (the standard rate for communications voice channels) and you took one second of samples, the data column vector is 8000 long.  The matrix is 8000 X 8000 square.  The zeroth row vector is 8000 entries of 1.

Row 1 is complex numbers all of magnitude one, with angles between them of 1/8000 of a circle: Δϕ=2π8000.

Row 2 is complex numbers all of magnitude one, with angles between them of twice that: Δϕ=2*2π8000.

Row 3 is complex numbers all of magnitude one, with angles between them of three times that: Δϕ=3*2π8000.

The rows continue to m=n/2, where the rotation is back and forth +1 and -1:

Δϕ=4000*2π8000=π.

That’s the Nyquist frequency.  Frequencies in the data vector cannot be detected there reliably unless they’re guaranteed to be cosinusoidal (pure real), as the sine part will be multiplied by zero.  Only use frequencies in the data vector that are below the Nyquist frequency.

(this started in late 19th century power engineering as the “cube roots of unity” problem.  The DFT matrix uses the various roots of unity up to half the sampling rate.  Try sketching the graphs of, say, 3 through 10th roots of unity in the complex plane)

The rows then continue to m=n-1, but if the data vector is purely real (the most common case), the upper set of results is the mirror image of the lower set.  It contains no new information.

The DFT matrix may be precomputed.  It didn’t used to be handled that way because the storage required scales with n-squared.  Storage is less of a problem now than it once was.

You may need only one or a small number of frequency bins.  If so: You do not have to do the entire matrix linear algebra problem.  You do not have to store the entire matrix.  Precompute and store only the required bins’ DFT vectors.  Doing the problem that way will be faster than doing the entire frequency domain vector with an FFT if only a small number of bins is required.

There is also a recursive maneuver for doing one bin called the Goertzel algorithm.  It requires only two complex numbers of storage and is therefore suitable for memory-tight environments such as Arduinos.  Again, if you need only one or a few bins, that method is faster than an FFT.

The FFT was originally defined only for “maximally composite” data sets, meaning data sampled into a vector that is a power of 2 long (16 samples, 256 samples, 1024 samples, and so on).  The advice was to “pad out” data sets that didn’t fit that to the next larger power of two, putting zeros in the extra slots so that the algorithm could be run efficiently.  FFT libraries now have enormous sets of special cases preprogrammed for less composite data sets, so don’t do the padding unless your data set happens to be a large prime number long.  If you’re writing an FFT algorithm as a homework programming exercise, it will be the simple one that requires the maximally composite data set.  Write the code, get your grade in the programming class, then use good quality libraries for real problems.  The Matlab pages are a good reference; so are Numpy’s.  Most of the libraries use The Fastest Fourier Transform in the West library (no, I’m not making this up).  It uses FFT along with the chirp Z-transform, which is another efficient algorithm for computing the DFT.  This gets complicated.  Use proven libraries for this and other linear algebra problems, not your own class coding exercise result.  Study some linear algebra; then study linear systems.  You will get no closer in your life to having a crystal ball that lets you see into the future than in examining linear dynamic systems and their linearized cousins.

The sampling rate for a DFT needs to be above the Nyquist rate, which is twice the bandwidth of the signal.  Note that the Nyquist rate is not twice the highest frequency in the signal (this is an oft-told myth or, more politely, misunderstanding).  The Nyquist rate is twice the signal bandwidth.  Bandwidth and maximum frequency are two different things.  Example: A radiotelegraphy (Morse code) signal at 20 words per minute has a bandwidth of about 30 Hz.  It might be transmitted on a carrier at 10 MHz, in which case the modulated signal’s maximum frequency is 10.000,015 Hz–much different from 30 Hz.  The Nyquist rate is 60 samples/second, not 20,000,030.  Either way, the signal being sampled has to be strictly bandlimited.  Aliasing is an unforgivable and unfilterable signal processing sin.  You need good analog filtering ahead of A/D conversion to avoid aliasing.  If you are using slow sampling to examine a high-frequency but bandlimited signal, note that the sampling aperture time (the amount of time over which each individual sample is taken) has to be fast enough to capture the high frequency signal.  And again, the filtering ahead of the A/D converter has to be able to reject out of band signals, or the aliasing will ruin the signal path.

Note also that the sampling theorem is an inequality.  Sampling has to be done at a rate greater than the Nyquist rate.  In practical applications, at least a 10% guard band needs to be added, i.e., the sampling rate has to be that much more than the Nyquist rate.  An example might be the high-fidelity audio band of 10 to 20,000 Hz used for music reproduction.  The linear band distance from zero to 10 Hz is too small to consider, so the maximum frequency is the practically same as the bandwidth, 20,000 Hz.  The Nyquist rate is 40,000 samples per second but the limiting filter cannot have perfectly steep skirts.  With a 10% guard band for practical filters, sampling may be done reasonably at 44,000 s/s.  The Compact DiskⓇ standard is in fact 44,100 s/s and the slowest recording studio digital audio standard is 48,000 s/s.  The history is complicated.

The data vector to be transformed is usually some string of real numbers representing the sampled signal.  In being sampled, the data are discrete in time (if they weren’t sampled, you’d be using a continuous Fourier transform and not reading this section).  That’s not necessarily a digital signal, although in contemporary use, it usually is.  “Sampled” means discretized in time.  “Digital” means discretized in amplitude.  They’re not the same thing.  Typical numbers might be 100 milliseconds of data sampled at 8000 samples per second, so 800 samples.  They might have been sampled by an analog-to-digital converter with 10 bit depth, so the numbers could be unsigned integers from 0 to 1023 (i.e., 210-1).  This will matter for the zeroth frequency bin, which will be the average of all the values (the DC component).  You may need to subtract off the average from each datum before beginning use of the data in some applications, such as a correlation.  The results for the frequencies above zero will not be affected.

The width of the frequency bins depends only on the sampling time.  One second sampling gives 1 hertz bins, regardless of sampling rate. 100 ms sampling time gives 10 Hz bins, and so on.  The highest frequency bin depends only on the sampling rate. Those two factors are independent–you can sample fast for a long time, fast for a short amount of time, and so on. 48,000 samples per second (standard for good quality audio) for one second gives 23,999 usable frequency bins that are one hertz apart, starting at zero (DC).  For a 0.1 second sample period, that sampling rate gives 2399 bins that are 10 Hz apart.  Faster sampling doesn’t give closer-together bins, only longer sampling periods can do that.

Other transforms

Fourier doesn’t entirely own the transform franchise.  Laplace transforms use sinusoids (imaginary exponential) multiplied by a real exponential to encode signals with initial conditions and decay or growth.  Short-time Fourier transforms, wavelet transforms and a whole pile of other methods constitute “time-frequency analysis,” which give information about signal frequency or feature detection as it changes over time.

There are also frequency-domain transforms that are computationally simpler than Fourier, such as the Walsh transform.  The Walsh uses square waves of values 0 and 1 instead of sinusoids. It’s computationally fast but it converges more slowly than Fourier transforms.

Gain, Bels, and decibels: Amplifiers vs. Antennas

“Gain” is another term that is used in two different ways, this time in one way for amplifiers and in another way for antennas. The first adds power from an external source; the second is passive and involves robbing radiated power from one direction to increase power in another direction.

Gain for amplifiers is the ratio of either power, voltage, or current output to one of those three input. The Circuits class teaches voltage gain in op-amp amplifiers; a gain of 1000 turns 1 millivolt input into 1 volt output, for example.  The gain is volts/volt, so it’s dimensionless.  Current amplifiers are common in analog integrated circuits and are defined similarly, with a dimensionless gain.  Amplifiers that turn amperes input into volts output have a gain in volts per ampere, which is the unit resistance; they are called transresistance amplifiers.  If your amplifier turns volts input to amperes output, it has gain in units of conductance, and such amplifiers are called transconductance amplifiers.

Power input transformed to power output plays slightly differently.  Example: An linear amplifier might be specified to have 50 ohm input impedance and 50 ohm output impedance; it will produce 20 watts output for 1 watt input.  It has a gain which may be expressed linearly as “gain of 20” or logarithmically as “gain of 3.0 Bels (which you will never see used) or gain of 30. decibels [dB] (perfectly fair game for accurate and precise communication to your colleagues)).  In general, then,

dB gain= 10 log10 (Pout/Pin)

Gain for antennas is the amount of power per solid angle in one specified direction divided by that which another antenna, either isotropic or dipole, would produce in its most favored direction.  This is almost always expressed in decibels and the reference to the isotropic or dipole antenna’s power is specified in a subscript “i” or “d”. You need to specify which, because the base power differs between them by 2.15 dB.

There is a subtlety to this distinction:  Conservation of energy must hold.  Amplifiers are active devices.  An amplifier’s gain comes from an external source.  Your stereo puts power into its speakers by extracting power from a wall outlet or battery.  An antenna is a passive device; its gain is measured relative to another passive device.  The total amount of power emitted by an isotropic antenna and by a gain antenna driven by the same transmitter is the same, only the directionality as a function of altitude and azimuth differs.

Yet more on decibels:  Decibel measurements always represent the common log of power ratios, computed with the output power of a system divided by its input power.  There are no “volt decibels” or “ampere decibels” because neither of those measures power, but there is a circuit analysis maneuver to coax the power ratios from voltage and current measurements.  If the power dissipation of the voltages (or currents) is measured into the same impedance, then power goes as the square of voltage and the log of the ratio may be multiplied by 2 to account for that.

dB gain= 20 log10 (Vout/Vin) (or I)

This is valid only if input and output resistances are equal!

If the two resistances are not the same, that difference needs to be accounted for–a voltage amplifier with a high input impedance and low output load has more power gain than the input and output voltages by themselves imply.  The log of the ratio of the impedances becomes part of the computation:

dB gain= 20 log10 (Vout/Vin) + 10 log10 (Rout/Rin)

This is a detail rarely gotten right.  Get it right.

The distinction between linear and geometric measurements is worth understanding:

xkcd: Optimal Bowling

Instrumentation Amplifiers

Another term that is used in two different ways.  You must know the context.

For most engineers involved in small-signal processing, the term simply means a differential input amplifier used in “an instrument.”  Such amplifiers usually are chosen to have high common-mode rejection ratio (meaning, they amplify the difference of its inputs and take that difference accurately, independent of the level of common added voltage riding on the two signals).  Such amplifiers are commonly realized as op amp circuits, with typical gains of 10 to 100.  Uses include buffering and amplifying high impedance sensor outputs such as those used in process control systems.  The exact type of amplifier might be as described in the next paragraph–but it might not.

In biologic signal processing and to an engineer involved in integrated circuit design, the term is used more narrowly and means a specific topology of three op-amps.  The circuit provides high input impedance and good common-mode rejection (i.e., it is a difference amplifier).  The gain is programmed with one resistor.  They are generally of lower frequency response than the op-amps would be alone.  They are quite useful; be familiar with the prepackaged versions, as they use laser-trimmed resistors on the chip and therefore are very well balanced for the common-mode rejection.  It’s not worth trying to make these yourself from three op-amp; your solution will be expensive, bulky, and of low precision.  Prepackaged ones are expensive as integrated circuits go because of the individual chips’ laser trimming, and you can’t duplicate that.

To avoid confusion in the terminology, consider using “ISA” to refer to the three-opamp topology.

Note that “instrument amplifier” is a different term, used in music–it’s an audio amplifier attached to an instrument.  It’s not the same thing as an “instrumentation amplifier.”

Units of Measurement

Be careful with units.  In fact, be very careful with units.  If your equation is adding quantities with different units, something is wrong.  Correct it.  Multiplying different units is legal (e.g., the vector quantity newton-meters of torque) and by extension, so is division (e.g., the scalar quantity meters/second of speed).  In that case: if units on either side of the equals sign aren’t the same, something is wrong.  Correct it.  If the argument of a polynomial or transcendental function (a function that is described by an infinite power series (Taylor series)), such as exponential, logarithm, and all the trigonometric functions) isn’t dimensionless, something is wrong.  Correct it.  Note that some areas of science are sloppy about this; an example is the chemist’s definition of pH.  That definition includes taking the logarithm of the concentration of hydronium ions, in moles per liter.  Can’t do it:  Concentrations carry units.  There has to be a standard concentration that the given one is divided by, and that isn’t stated in the usual pH definition.

Note that the argument of a dirac delta function (impulse), y=δ(x), is the inverse of the units of the function itself (the independent variable), by the function’s scaling property.  The impulse function is not a transcendental function, and it has no Taylor series representation!  This is at best weird and I recommend clapping your hands to produce impulses and then not worrying too much about it.

Angle measurements

One circle per circle

Sounds obvious, but it’s the angular measurement used for cycles per second, or hertz. It’s also used in rotating machinery but usually with a non-SI time as “rotations per minute” or RPM.   Since “cycle” is dimensionless, the dimensional unit for hertz is sec-1.  This is the same as the dimensional unit for radians per second, so be careful to catch the context.  The full units don’t mix: You can’t add circles and radians, so you can’t add hertz and rad/sec.  There are bad puns regarding dolor to made here; they’ve been made, we’ve all heard them, keep them to yourself.

360° per circle

This one is common for everything except frequencies (there’s no reference to the AM radio station at 360,000 kilodegrees per second on your radio dial, it’s at 1000 kilohertz).  It does show up in some odd settings; a professional baseball player’s eyeball rotation rate is something like 700° per second as the pitched ball approaches the bat.,  That rotational rate is not sustained for more than 30° or so(!).  Do not refer to that as 2 rotations per second outside of horror film script writing.

Degrees is the number of angular units customary in ancient Babylonia,  traversed on a circle by going 1 revolution. The origin of the definition of a degree as 1/360th of a circle is lost to the mists of time, but likely is some combination of 360 being close to the number of days in a year (and so the fixed stars move in the sky approximately one degree per day).  360 also has a conveniently large number of divisors.  LIkely there are other reasons.  Make your own guesses.

2π radians per circle

Radian measures are used commonly in engineering settings for frequency; radians per second is fair game and should be well understood. An AM radio station might be “1000 on your radio dial,” or 1000 kHz or 1 MHz.  That’s 6.28 million radians per second.  By standard SI use, 6.28 megarad/sec should be an acceptable locution but I’ve never heard magnitude prefixes used with radians.  Line frequency of 60 Hz may be stated as 377 radians/second to precision better than that  of the line frequency, usually given as 500 parts per million or 0.05%. Work it out on a calculator if you don’t see that one..

Radians is the native angular unit for all trigonometric functions.  It represents the length of unit circle circumference swept by the given angle.  This is often handled in modular form, so know the “wrapping function of x,” which is x mod 2π.  All the trig functions are periodic in 2π, so graphs of things like signal phase can look sawtoothed instead of smooth because the wrapping function has been invoked.  Whether they are “really” sawtoothed or linear is a matter of philosophy and drawing convenience; be careful in interpreting such graphs.

2π radians is exactly the same thing as 360 degrees, so π radians is 180 degrees, and one radian is 180/π or 57.30 degrees.  It’s useful to know that a radian is in the general vicinity of 60 degrees (within 5%), and a degree is around 0.0174 radians.

400 Gradians (“grads” or “gons”) per circle

Essentially metric degrees–the measure arose during the French Revolution and is still used in Europe for artillery and for surveying.  You may use them for aiming inter-dormitory water guns during undergraduate years; you will do less of that and soak fewer grads after commencement.  Gradians have the convenience of being 100 per right angle.  Calculators now mostly have only radians and degrees for trigonometric functions, but they included gradians through perhaps the 1990s.  HP-11C calculators were the best calculators ever made and we will not entertain arguments about that.  They had gradians.  Learn RPN. Modern algebraic calculators are silly. Search Ebay for an HP-11C and a Kueffel and Esser Log-Log Decitrig slide rule and be a real engineer.

Some power of two per circle

This is used in direct digital synthesis (DDS) systems.  One unsigned integer variable represents the circle; overflow represents a complete rotation of the phasor.  A very good discussion by Cornell professor Hunter Adams may be found here. He uses 232  for the divisions of the circle, convenient for being a “large int” in most languages. From that: “The observation upon which the direct digital synthesis algorithm is based is that a variable overflowing is isomorphic to one rotation of a phasor.” Cheap and easy to use in hardware: AD9850.  There are more modern versions of that integrated circuit with larger phase accumulators, typically 240.  Prof. Hunter’s software solution for Raspberry Pi Pico is good for audio frequency synthesis.

Frequency Units

Hertz is an SI unit for one waveform cycle per second.  It has units of inverse seconds.  It displaced the unit “cycles per second” because there isn’t a unit “cycle,” and computations need time times frequency to be unitless.

Radians per second is also an SI unit.  It also has units of inverse seconds.

Degrees per second is fair game but not SI.  Be careful you’re not following a summer heat wave instead of a signal.

Rotations per second is hertz.  Rotations per minute (RPM) is not hertz and not an SI unit, but it’s the standard in rotating machinery such as automobile tachometers.  Try pointing an optical tachometer at a fluorescent light tube, then explain why it reads 7200 RPM.

Other Units

SI Units (this section needs some fill-in; time, voltage, resistance, capacitance, inductance, luminous flux, magnetic flux vs. magnetic field, etc.)

Ampere is an SI base unit; coulomb is not.

1 ampere is now defined as 1 coulomb of charge going past a given point per second.

Coulomb is now defined via the elementary charge, which is defined in SI exactly as

e ≡ 1.602176634×10−19 coulombs.  The coulomb, in turn, is defined as

exactly 1/(1.602176634×10−19) elementary charges.  Note how different Faraday’s constant (that numerator) is from Avagadro’s number; this in part explains the difficulty of building good batteries.

Kirchoff’s Laws

Note that Kirchoff’s voltage and current laws both apply only under specific circumstances.  Your introductory circuits textbook mentioned this but your professor, I can guarantee you, did not.  Those circumstances hold in very many cases, but not in all; be able to look at your circuit or electrical system and make an accurate determination whether or not KVL and KCL may be applied.

KCL is a statement of conservation of energy.  If a circuit has an external energy source such as a linked, changing magnetic field (i.e., if you have unwittingly (or maybe wittingly, I do not know what nefarious circuit deeds you are up to)) made your circuit the secondary of a transformer, KCL will seem to fail.  For that matter, a transformer is always an apparent failure of KCL.  That is the physics inductive coupling, and the engineering of it is to transfer energy from one place to another without a circuit connection.  The same is true for external, varying electric fields pumping charge in your circuit.  You’re not including the entire circuit in your analysis if you don’t consider the external fields.

KVL is a statement of conservation of charge.  It will seem to fail if there is some external source of charge for the circuit or if device initial conditions are not charge-neutral.  Capacitive coupling can do this; it is the dual of the KCL failure.

Both KVL and KCL will similarly seem to fail if circuit dimensions are not much smaller than signal wavelength.  Invoking KVL and KCL, you are not considering circuit “nodes” and “meshes” actually to be transmission lines.  If something in your circuit is larger than about 5% of a wavelength for signals being carried, make sure your circuit analysis is an adequate approximation of the real physics.

Linearity and first-order-ness

A basic circuits course presents resistors, capacitors, and inductors as “the linear elements.”  They are described by an algebraic equation (resistors) or a first-order ordinary differential equation (capacitors and inductors).  They seem to be manufactured by mathematicians.

The problem is that nothing real is linear or is fully described by a linear, first-order ordinary differential equation.  Every one of those two-terminal devices will have at least some resistance, some capacitance, and some inductance, so they’re not “really” first-order.  Similarly, they will have temperature dependence, frequency dependence (say, from conductor skin effect), aging, dimensional change from the induced electric and magnetic fields, and so on, so they’re not “really” linear.  If you’re lucky, the part labeled “resistor” will have more resistance than anything else at the frequency, power level, humidity, and other conditions in your circuit and your circuit’s environment, and it will stay put at its labeled value long enough to be useful in your application.  Ditto for the capacitors and inductors.  Each of the three will have a self-resonant frequency or even multiple self-resonant frequencies.  Each of those frequencies may be from parallel or series resonance.  Capacitors will have “effective series resistance” and temperature coefficients, aging, and the list goes on.  When your half-watt resistor burns partially open because you made it dissipate one watt for a few seconds, it will not be linear, of low order, or time invariant, at least not until it cools off.  See if the color code is charred over, that and the burning phenolic smells are good first signs of component illness.

That the parts are not defined by an ordinary differential equation is because they occupy some nonzero amount of space.  If they are small compared with the smallest wavelength signal They will be acting at least in part as transmission lines or antennas.

Again, only ideal capacitors are first-order.  SAFETY NOTE: Pay attention to “dielectric absorption.”  You may find yourself working with high-voltage capacitors that store significant energy, such as in electric power settings.  They are very much not first-order devices.  They can retain internal energy after their terminals have been shorted together for periods of time ranging past days into years; when the shorting bar is removed, the energy manifests as spontaneously rising voltage across the terminals.  This can be lethal.  Use that supplied shorting bar: Keep capacitors with energy storage of more than 5 joules or so shorted whenever they’re not in use in a circuit.  Note that this is a very real problem when working on cardiac defibrillators in biomedical and clinical engineering settings–those capacitors store 400 joules at high voltage.

FURTHER SAFETY NOTE: Be aware of high voltage capacitors that are reasonably first order but are also of reasonably high quality and low leakage.  They will hold their charge and voltage for months.  Add leakage in the form of a “bleeder resistor” across such supplies and accept that they will get hot (the i2R losses can be high) and prone to failure.  Add those resistors as safety devices, then when you open a transmitter cabinet to service the circuit, assume that the bleeder may have failed and check it before putting your hands onto the tube’s B+ line.

Environmental sensors (“transducers”) of various kinds usually manipulate non-constant values of R, C, and L.  A capacitive microphone (“condenser microphone,” after the old term for capacitor) uses the varying sound pressure to move one plate of a capacitor.  The capacitance changes with time; that change is the measure of instantaneous sound pressure.  Name your next rock band “The Farad Variers” or some such.

To avoid giving stray potential differences a discharge path through your heart, which I add for completeness is bad for your heart,, learn to work on live circuits with one hand only.  Keep the other hand behind your back so you don’t reflexively grab with the other hand.  Surprisingly low voltages can produce enough current to induce cardiac dysrhythmias, especially through damp skin.

Some references on electrical lab safety:

Safety (arrl.org)

Learn to read manufacturer’s specification sheets, even for seemingly insignificant parts such as capacitors, resistors, and inductors.  Otherwise quite good engineers have a touching faith in, for example, 1 watt resistors being able to dissipate rated power when the ambient temperature is that of inside an automobile engine.

All of the first-order components are used in modeling loads.  Not all loads are heat-dissipating resistors.  A home furnace may be; it takes in electrical power and emits thermal power.  It may not be very good demonstration of Ohm’s law as its terminal ratio of voltage to current will vary with its temperature and that variance is the whole reason you bought and installed the furnace.  An antenna takes in electrical power and emits electromagnetic waves that carry the power away.  It has a ratio of voltage to current at its terminal that is at least partly real–but it could be made of ideal, perfectly conducting wire and still act as a radiator of electromagnetic energy.  The terminal resistance is “radiation resistance” rather than “ohmic resistance” and does not cause heating.  It does cause radio!

Beat Frequency

Speaking of linear vs. nonlinear systems–the term “beat frequency” is used differently in acoustic and radio settings, with some overlap.  You’ll have to get from context which use is meant.

Acoustics uses the linear meaning:  Two signals of different frequencies are added and the constructive and destructive interference pattern alternates at a frequency equal to the difference of the two frequencies.  It’s the effect that is used for tuning musical instruments (the beat is reduced to zero), and it’s the annoying sound of out-of-tune instruments (a fluttering “wah-wah”).  No new frequencies are produced.  It is the equivalent, however, of a tone with frequency at the average of the two tones’ frequencies, varying in amplitude at the rate of their frequency difference.

Radio engineering uses the nonlinear meaning.  Two signals of different frequencies are multiplied together, and two new frequencies are generated.  The new frequencies are at the difference and sum of the two original frequencies and the original one vanishes.  An old locution in radio receiver design is the “beat frequency oscillator,” which generates a radiofrequency signal local to the receiver, adds it to the incoming signal, and uses a diode detector to generate a difference pitch.  This is useful in permitting continuous wave reception in an AM receiver, for example.  It is a primitive form of a “product detector,” and does in fact take the product of the incoming signal and the locally generated one.

Solenoid

Another term used in two different ways.  Know the two definitions and understand them contextually.  The etymology is interesting.

In electrical engineering and in physics, a solenoid is a helix of wire used for its magnetic properties.  A solenoidal inductor is one wound as a helix with a (usually) straight axis.  They are easy and inexpensive to make, and their inductance is well described by several empirical formulae.  Their magnetic field lines exit the “tube” of the helix at one end and loop back around to the other, so they are prone to inductive coupling of one circuit to another.

In mechanical engineering, a solenoid is a linear motor made up of a solenoidal inductor used as an electromagnet and a ferrous actuator that moves when the current is turned on and off.  They’re very common.  The naming is confusing.  Even worse is that there are “rotary solenoids,” with a shaft that moves through some fixed angle when the device is energized.  Some are monostable, some are bistable.

Note that toroidal inductors are more or less solenoidal inductors wound into a loop, so that the magnetic field lines wrap back on themselves without having to exit the loop.  They are self-shielding in the sense that their magnetic field is minimal outside the helix, but they are more difficult and more expensive to make than are solenoids.  Those are not reasons to be scared of them; try winding a few of both kinds and verifying the inductance formulas on an inductance meter.  Try building a radio!

Ground

Yet another term used in different ways, often loosely and incorrectly.

Earth ground

Chassis ground

Signal ground and RF (radiofrequency) ground

Motor lore

This is another area of electrical engineering that has arcane and sometimes confusing vocabulary that has accreted over the past 150 years.

The term servo motor is used in several different ways.

The older use is DC motors with a reasonably well-defined, reasonably linear current/torque and voltage/RPM (angular velocity) relationships.  Control systems that use feedback loops of torque, angular velocity, and angles or positions use dynamic models of the mechanical and electronic systems and are now well-supported by specialized analog integrated circuits.

Quite different are servo motors with pulse-width-modulated position inputs.  The PWM input is only a command, not a modulated current input to the motor. These are sometimes called “model airplane servos” but in fact exist up to enormous industrial size actuators, both linear and rotational.  The pulse-modulation scheme is the same for all of them and are they are a very useful tool, worth knowing about.  Some have torque feedback.

“Slip” has two different meanings in AC motors:

- In synchronous motors, load causes phase slip in the shaft.  In steady-state, the rotation velocity stays locked to the driving signal but the angle between the two is a monotonic function of the load.

- In induction motors, load causes angular velocity (RPM) slip in the shaft.  The rotation velocity is not locked to the driving frequency and the shaft slows with increasing load.

Phase slip and velocity slip are two quite different things, akin to phase modulation vs. frequency modulation. Know the difference!

Electromagnetic Wave Polarization: Linear, Circular, Elliptical

All three of these polarizations are vector sums of two perpendicular linear polarizations, usually called vertical and horizontal (older parlance: “ordinary” and “extraordinary” from 19th century crystallography) but note that space doesn’t have a horizon reference.

The general case is elliptical polarization.  An ellipse with eccentricity of zero is a circle, and that corresponds to circular polarization.  Eccentricity “nearly 1” is a straight line (1 exactly denotes a parabola; the foci have gotten infinitely far apart). That denotes linear polarization.  The full span of eccentricities is seen in received signals that were launched as linearly polarized or circularly polarized, because of propagation effects; for the usual radio antennas and therefore for transmitting, only linear and circular polarization are deliberately designed into a system.

Circular polarization is handed (Greek “chiral” χείρ (kheir), "hand." Hands are chiral; feet sometimes less so). Handedness is the same property that a helix or machine screw has in that left-handed and right-handed ones are two different things, related by a reflection as in through a mirror.  A right-hand receiving antenna will not receive any signal from a left-hand incoming transmission.  This is done deliberately in engineering design to suppress reflected signals. It shows up in some biologic settings as well; light reflected off Japanese beetle wings is an example, and the beetle eyes are of matching chirality.

Electrical engineers and physicists/opticians use opposite definitions for the right- or left-handedness.  The EE standard was supposed to be the same as the much older one from optics but it got reversed in some committee.  Such is life; Benjamin Franklin got the sign of electric fluid motion wrong, too, and we’re stuck with electrons moving one direction and current the other.  You need to know both.  Electrical engineering views the signal going away from the source; physics views it approaching the receiver.

Linear polarization is named for the direction of the electric field, not the magnetic field.  A horizontal wire antenna will produce or receive a horizontally polarized signal.  Magnetic slot antennas are therefore polarized perpendicular to the slots.  This takes some getting used to.

Slide Rules, Log Tables, and a “Feel for Numbers”

Really.  Digital computers are good, but so are analogue ones.  The “analogy” in a slide rule is position of the slide to the real number line.

Logarithms map geometric space into linear space.  The most common logarithmic graph paper uses logs base 10 (“common logs” as opposed to natural logs, denoted as ln. Note that in some texts, “log” means natural log, too; use “ln” and “log10” in your own writing to remove ambiguity.  There is some slop in this document and most others, get used to it).  Every factor of ten gets the same amount of distance along the logarithmic number line.  Note that numbers from 1.0 to 2 take up about a third of each decade, and 3 and π are about in the middle.  Those are both just below (10), which is the middle of the decade (log((10)=12log(10)).  Pi equals square root of ten for large pi or small 10.  Remember that.

Slide rules divide the real number line logarithmically.  Adding distances by moving along the resulting line in logarithmic increments is the equivalent of multiplying.  Get an inexpensive slide rule or print one from any of a number of web sites that have such things and try multiplying numbers together on the C and D scales.  Note how multiplications that involve a carry wrap into the next cycle of logarithms.  This is not the place for a full discussion of how to use slide rules, but they’re fun and will offer a visual display of number magnitudes that you may find useful.

Also try applying the rules of logarithm arithmetic to some actual numbers.  Pick a couple of numbers to multiply together and use your calculator to find their common logs.  Examine those logs, look at the integer part and the fraction part separately to get some understanding of how the logs construct the number and its magnitude, and add them together.  Take the antilog of that sum (use the 10x key) and verify that the result is the product of the two numbers.  Take a moment to pity your pre-calculator professional ancestors for having to do this for all their number handling–but respect the sense for numbers that they had and that you and I likely don’t.

Op-Amps

The ideal op-amp approximation will get you a long ways.  Here is a list of ten fundamental op-amp circuits.  I would add the instrumentation amplifier, as discussed above, but it’s not a bad list.  Remember that differentiators are noisy, that integrators can be unstable (add a resistor across the capacitor), and that there are times when you need to use non-ideal models.

Dualities

Section to be worked on.

Here is a partial list of electrical dualities:

voltage – current

parallel – series (circuits)

resistance – conductance

voltage division – current division

impedance – admittance

capacitance – inductance

reactance – susceptance

short circuit – open circuit

Kirchhoff's current law (KCL) – Kirchhoff's voltage law (KVL)

Thévenin's theorem – Norton's theorem

### Miniaturization: What breaks when you go small?

Class 2 MLCCs (multi-layer ceramic capacitors) derate to a lower effective capacitance as you approach a dc bias close to the cap’s max voltage. The curve can look something like:

Depending on the application, this effect can really matter! The curvature gets to be more aggressive when going smaller. The large reason being that as the capacitor gets smaller, the choice in dielectrics available go down. For certain high capacitance small footprint capacitors, like 0201s, class 2 dielectrics are the only available dielectric able to achieve the high capacitance.

Textbooks

A few to have on your shelf:

The ARRL Handbook for Communications Engineers, whether or not communications is entirely your thing.  The general electronics knowledge presented in it is excellent, and it excels in basic radiofrequency electronics.  Plus, communications is fun, get your amateur radio license and try it out.  Get your amateur radio license. Every engineer should have an amateur radio license.  For that matter, get your commercial radio license, too; we recommend the Gordon West study guide for the GROL (General Radio Operator License) and Ship Radar Endorsement.  The Radiotelegraph Operator license will put you in the rarest licensure category of all (only 182 issued as of this writing; you will be the only nerd on your block to have that license), and we can administer all of these tests.  The book is updated annually; the first edition was published in 1926.

Paul Horowitz, Winfield Hill, The Art of Electronics, Cambridge University Press 2015.  This is a little dated but has everything you might want for general laboratory electronics knowledge–except, perhaps radiofrequency design.

William H. Press, Numerical Recipes in C++: The Art of Scientific Computing, Cambridge University Press 2009. This is roughly the software equivalent of The Art of Electronics.

Barry Dorr, Ten Essential Skills for Electrical Engineers, IEEE Press.  Use this as part of your preparation for interviews and for the Professional Engineer license.  Get your engineering license.  You are going through the trouble of getting an accredited engineering degree; use that degree to attain licensure.

Think about that one–the Taylor expansion of a function such as exponential contains all nonnegative integer powers of the argument.  If the argument has units, the expansion has units of varying power, and the terms cannot be added.  The argument must be dimensionless.

after Jean-Robert Argand, the Paris bookstore manager and amateur mathematician who rediscovered the complex plane, a few years after it was described by the Danish-Norwegian mathematician Caspar Wessel.  Argand certainly deserves to have his name remembered, and more bookstores deserve such managers, but choose some complex libation and raise a cup of kindness to Wessel, too.

For example, the usual use for an EE’s impulse function in time (δ(t))is to find volts, given as a function of time, at one particular time.  The impulse function then integrates over time to be dimensionless, so the function’s own units must be inverse time.  As we said: Weird.

44,100 s/s is the Compact Disc Digital Audio rate; 96,000 is used in professional recording studios; 192,000 is used less commonly in professional and nature-sound recording settings.  That last is high enough for bat sonar detection; not too far from the high notes here: "Der Hölle Rache kocht in meinem Herzen," 192 ks/s will not alias that aria, either, and that’s the sampling likely used for the near-ultrasonics this: Mozart "Der Hölle Rache kocht..." (Patricia Petibon) - YouTube Note that the Petibon’s Herzen are unrelated to the Heinrich’s Hertz.  Once more, with feeling and captions: https://youtu.be/lJJW0dE5GF0

As good as any: Make a Simple Paper Slide Rule : 10 Steps - Instructables

Fraktur - Wikipedia.  “Blackboard bold” is easier to write by hand and sometimes shows up in print: https://en.wikipedia.org/wiki/Blackboard_bold

Note that if the sinusoids are of constant frequency, the phase difference does not change with time.

It’s counterclockwise by definition.  Charles Steinmetz, the inventor of phasors at General Electric, patterned their motion after the rotational direction of the circumpolar stars in the northern hemisphere.

We have been joking in various CWRU classes about our sponsor, Yost Manufacturing, and its slogan: “We’re ideal! And first-order!”  This came to have less meaning in 2023 when Yost building, home of CWRU’s math department, was razed; the math department underwent fission and moved to various quarters. AC8XY is preparing a new slogan and a matching logo.

Omitted is memristors: Memristor - Wikipedia  They’re not for sale–yet.

https://en.wikipedia.org/wiki/FFTW

From the Wikipedia page on “Argument of a Function,” :  The use of the term "argument" in this sense developed from astronomy, which historically used tables to determine the spatial positions of planets from their positions in the sky (ephemerides). These tables were organized according to measured angles called arguments, literally "that which elucidates something else."

Nicol prism - Wikipedia

Interesting history to why decibels are used rather than bels; has to do with early telephone line measurements and a need to preserve approximate computation forms.

https://en.wikipedia.org/wiki/It%27s_Five_O%27Clock_Somewhere

https://en.wikipedia.org/wiki/Goertzel_algorithm

https://en.wikipedia.org/wiki/Chirp_Z-transform

Vocabulary study is a good thing: https://www.oxfordlearnersdictionaries.com/us/definition/american_english/polysemous
It’s a little like overloading an operator in computing.

In signal processing contexts, these will be called the “I” and “Q” parts, for “in-phase” and “quadrature.”

https://en.wikipedia.org/wiki/LAPACK

At some frequencies, the system may multiply the input signal by zero, so the concepts of phase and frequency are lost.  Causal linear systems can only do that at specific points of frequency, not over a range..  Relaxation of causality offers considerably more flexibility.  The radio club will not fund senior projects that involve the construction of time machines.

https://psycnet.apa.org/record/2021-33856-001

Nick Fogt and Mason Clutter, “Do Baseball Batters Keep Their Eye on the Ball?” Baseball Research Journal Fall 2024, https://sabr.org/journal/article/do-baseball-batters-keep-their-eye-on-the-ball/

The goofiness of this is worth reading about: https://en.wikipedia.org/wiki/Circular_polarization#Uses_of_the_two_conventions

The modulo operation is mostly defined on integers only but there’s nothing wrong with using real modulus, dividend, divisor, or remainder.

Record temperature rates of change, from https://en.wikipedia.org/wiki/List_of_weather_records#Temperature

- Greatest 2-minute temperature increase: 27 °C (49 °F), from −20 °C (−4 °F) to 7 °C (45 °F); Spearfish, South Dakota, on 22 January 1943.

- Greatest 24-hour temperature increase: +57 °C (+102.6 °F), from −48 °C (−54 °F) to 9 °C (48 °F); Loma, Montana, on 15 January 1972.

- Fastest 2 minute temperature drop: 15.0 °C (27.0 °F) at Sydney Airport, Australia, on 23 January 2010, from 41.0 °C (105.8 °F) at 3:11PM to 26.0 °C (78.8 °F) at 3:13PM.

Americas.  Look for 6000 RPM in Europe.

The word "solenoid" comes from the French term "solénoïde," which was coined by the physicist André-Marie Ampère in 1823. It is derived from the Ancient Greek word "σωληνοειδής," meaning "pipe-shaped," combining "σωλήν" (sōlēn) for "pipe" and "εἶδος" (eidos) for "form" or "shape."

https://www.merriam-webster.com/dictionary/two%20left%20feet

This antenna has a horizontal slot and is thus vertically polarized: https://www.w6nbc.com/articles/2016-03QSTdishslot.pdf